SPECIFICATIONS

BI FLEX SYSTEM

Supply and installation of waterproof elastomeric terpolymer tape combined with a two-component epoxy adhesive for the treatment of joints and cracks.

The system must have a minimum thickness of 1 mm depending on the type of application chosen.

The product must be used for:

- To apply a waterproof sealing on joints and cracks

- To seal construction joints

- Connect adjoining surfaces of joints, structures in general or prefabricated elements

- Waterproofing of tanks, canals, swimming pools and structures intended for containing water, including drinking water

- Sealing of discontinuous elements for water and/or air tightness

The material must have the following characteristics:

Specification, Test method and 0.5 mm thickness - Positive pressure water

Concrete peeling resistance (ASTM D 903 - 180°): > 0.5 kN/m

Positive pressure water resistance on open joint* (Int Method): 2 cm: 3 bar

Negative pressure water resistance on open joint: 1 cm: 1 bar

Adhesion to damp concrete (UNI EN 13578): > 2,0 N/mm²

Specification, Test method and 1 mm thickness – Positive/negative pressure water

Concrete peeling resistance (ASTM D 903 - 180°): > 1.5 kN/m

Positive pressure water resistance on open joint* (Int Method): 1 cm: 3 bar;2 cm: 3 bar

Negative pressure water resistance on open joint: 1 cm: 1 bar;2 cm: 0.5 bar

Adhesion to damp concrete (UNI EN 13578): > 2.0 N/mm²

Specification, Test method and 1.5 mm thickness – Positive/negative pressure water

Concrete peeling resistance (ASTM D 903 - 180°): > 1.5 kN/m

Positive pressure water resistance on open joint* (Int Method): -

Negative pressure water resistance on open joint: 2 cm: 1 bar

Adhesion to damp concrete (UNI EN 13578): > 2.0 N/mm²

* Tests were carried out by testing an overlap area bonded with BI MASTIC sealant

Feature, Certifying body, Test method, Certified performance

Radon diffusion coefficient - BI FLEX, Czech Technical University In Prague, ISO/TS 11665-13: 2.5 E-11 m²/s

Radon diffusion coefficient - BI FLEX, overlap sealed with BI MASTIC, Czech Technical University In Prague, ISO/TS 11665-13: 1.9 E-11 m²/s

Specification, Certifying body, Certification

Suitability for contact with drinking water. Determination of global migration Ministerial Decree 174 of 06/04/2004, SOCOTEC ITALIA SRL: Report n° LF53030/23 - LF53031/23

Suitability for contact with drinking water Determination of specific release Ministerial Decree 174 of 06/04/2024, CHELAB SRL: Report n° 17/000244151

EN Specifications n. EY- EW | CP | 04 | 00 | W | 01/26

SPECIFICATIONS

BI FLEX SYSTEM

as well as Bi Flex System Volteco or a product with equal or superior characteristics.

The technical data must be supported by test certification issued by an accredited official laboratory and/or be subjected to quality control according to ISO 9001.

For further details on the individual products and installation specifications, refer to the relevant technical data sheets which can be downloaded in the updated version on the website www.volteco.com.

EN Specifications n. EY- EW | CP | 04 | 00 | W | 01/26