WT CONSTRUCTION

Supply and installation of hydro-expansive profile based on Amphibia copolymer in EPDM rubber, reinforced in the center with a co-extruded steel mesh that stabilizes and directs the volumetric expansion.

The product must be used for:

- To seal vertical and horizontal concrete construction joints

The material must have the following characteristics:

Certification according to ETA 24/387

Specification, Values

Specific weight: 1.02 g/m3

Application temperature: -15°C +50°C

Colour: black

Feature, Test method, Result/Classification

Reaction to fire, EN ISO 11925-2 – EN 13501-1: Class E

Dimensions	WT C 20.10, EAD 2.2.2:	20x10 mm (± 10%)

WT C 25.15, EAD 2.2.2: 25x15 mm (± 10%)

Impermeability under end-use conditions, EAD 2.2.3: 20 m water column / waterproof up to 5 bar

Swelling pressure		WT C 20.10, EAD 2.2.6:	1.2 N/mm²

WT C 25.15, EAD 2.2.6: 1.4 N/mm²

Expansion in saline water (3%): > 100%

Hydraulic seal on 5 mm crack with expansion of 100%: No passage up to 150 kPa

Feature, Test method, Test liquid, Time until constant mass is achieved, Change in mass

Expansion in contact with different liquids,	EAD 2.2.4,	Deionised water,	25 d: 1004 %

Alkaline solution,	14 d: 455 %

Acid solution,	24 d: 474 %

First expansion cycle, 	EAD 2.2.5,	Deionised water,	16 d: 1794 %

Reversibility of the expansion process	Alkaline solution,	16 d: 479 %

Acid solution,	23 d: 503 %

First expansion cycle,	EAD 2.2.5,	Deionised water,	14 d: 1855 %

Reversibility of the expansion process	Alkaline solution,	14 d: 419 %

Acid solution,	21 d: 436 %

- the expansion process is reversible 
- the expansion process begins immediately

EN Specifications n. BF01 | CP | 02 | 01 | W | 07/26

as well as Wt Construction Volteco or a product with equal or superior characteristics.

The technical data must be supported by test certification issued by an accredited official laboratory and / or be subjected to quality control according to ISO 9001.

The product must have the CE marking and be used according to the manufacturer's prescriptions.

For further details on the individual products and installation specifications, refer to the relevant technical data sheets which can be downloaded in the updated version on the website www.volteco.com.

EN Specifications n. BF01 | CP | 02 | 01 | W | 07/26